The Saturn project has grown to include a number of USB serial devices interfaced to the Raspberry PI ARM processor. The executing software *typically p2app or piHPSDR) need to know which device is which to connect to it.

These devices are mostly but not exclusively Arduinos. All devices that I’ve created respond to a “ZZZS” message and reply with their identify, hardware an software version number. This way software can be confident that they have found the right device. However different parts fo the software will be trying to connect to different devices, so some convention for device name is appropriate.

A complication is: several such devices use the same physical Arduino type. That complicates naming. We have settled on a “physical address” method to name devices; it takes their location in the USB interface tree. That means we will need consistence of where devices are plugged in. The arrangements are different for different types of ARM processor.

udev rules are being used to assign suitable addresses.

# udev Connection path identification

The current USB ports in Saturn radios can be identified by the KERNELS setting in udevadm. It identified serial ports as follows:

| Port | Radxa CM5 | Raspberry pi CM5 | Used For |
| --- | --- | --- | --- |
| Rear panel, top | 1-1.2 | 5-1.2 | Mouse/keyboard/memory/etc |
| Rear panel, bottom | 1-1.1 | 5-1.1 | Mouse/keyboard/memory/etc |
| Front panel, external | 1-1.3 | 5-1.3 | Mouse/keyboard/memory/etc Ganymede (G2-1K) |
| Front panel PCB, rear | 1-1.4 | 5-1.4 | G2V2 front panel G2V1 front panel adapter Aries Ganymede (G2-1K with front panel) |

If we consider any devices beyond those in use today, a USB hub on the rear of the front panel PCB will be needed (which could be a hub on a modified front panel board itself).

# Device Types

As at January 2026 the following types of device are known:

| ARM device | Device | Module Type | Interface location |
| --- | --- | --- | --- |
| Radxa CM5 | G2V2 front panel | Arduino Nano every | USB, rear of front panel PCB |
| Radxa CM5 | G2V1 front panel adapter | Waveshare RP2040-pizero | USB, rear of front panel PCB |
| RPi CM4 | G2V2 front panel | Arduino Nano every | ARM serial on 40 pin connector |
| RPi CM5 | G2V2 front panel | Arduino Nano every | ARM serial on 40 pin connector |
| RPi CM5 | Ganymede (G2-1K) | Arduino Nano every | USB, the port that would go to front panel external. |
| RPi CM5 | Aries (experimental) | Arduino Nano 33 IoT | USB, rear of front panel PCB |
| Radxa CM5 | Aries (experimental) | Arduino Nano 33 IoT | USB, rear of front panel PCB, needing a hub |

G2-1K radio at initial release will have no front panel. A front panel may be added for later version. It is assumed that all shipped radios would have a Raspberry pi CM5. A USB hub would be required for front panel and Ganymede microcontrollers.

Further devices to come include Aries ATU, using Arduino Nano 33 IoT. For RPi CM5 that could connect to the front panel rear USB. For a Radxa CM5 radio it would have to connect to that same connector via a USB hub.